## FIXTURES

### Cyc Strip

Housing shall be constructed of aluminum and steel with gray hammertone and high temperature finish.

Optical Train consists of a recessed single contact double ended base tungsten halogen lamp, specular random patterned reflector, and an easily accessible socket assembly for lamp replacement.

Socket assembly shall be steatite insulated, UL listed, rated for 600 volts, 3000 watts, 200oC continuous operation.  Rated lamp seal temperatures shall not be exceeded.

For mounting, unit shall be provided with a heavy steel trunion plates for floor mounting and painted malleable iron C-clamp for up to 2" ID pipe.

Unit shall be provided with 36" 3-wire braided glass leads with connector as specified by catalog number.  The entire unit shall be UL listed and labeled.

Provide the Following:

Qty.    Catalog No. 	Description

#	108-472		1 Light, 1 Circuit with Stage Pin Connector

#	108-474		1 Light, 1 Circuit with Parallel Blade Connector

#	108-476		1 Light, 1 Circuit with Locking Connector

#	108-492		2 Light, 2 Circuit with Stage Pin Connector

#	108-494		2 Light, 2 Circuit with Parallel Blade Connector

#	108-496		2 Light, 2 Circuit with Locking Connector

#	108-452		3 Light, 3 Circuit with Stage Pin Connector

#	108-454		3 Light, 3 Circuit with Parallel Blade Connector

#	108-456		3 Light, 3 Circuit with Locking Connector

#	108-592		4 Light, 4 Circuit with Stage Pin Connector

#	108-594		4 Light, 4 Circuit with Parallel Blade Connector

#	108-596		4 Light, 4 Circuit with Locking Connector

#	108-532		6 Light, 3 Circuit with Stage Pin Connector

#	108-534		6 Light, 3 Circuit with Parallel Blade Connector

#	108-536		6 Light, 3 Circuit with Locking Connector

#	108-562		9 Light, 3 Circuit with Stage Pin Connector

#	108-564		9 Light, 3 Circuit with Parallel Blade Connector

#	108-566		9 Light, 3 Circuit with Locking Connector

#	120-020		Diffusion frame

#	126-014		Safety frame

#	138-059		Safety cable

#	158-009		Pipe and Floor Mounting Assembly

#	158-014		Yoke & C Clamp Mounting Assembly 								for 1-light Cyc unit

#	158-016		Yoke & C Clamp Mounting Assembly 								for 2-light Cyc unit

#	158-017		Yoke & C Clamp Mounting Assembly 								for 3-light Cyc unit

#	176-019		500 watt lamp, 2900oK EJD

#	176-022		1000 watt lamp, 3200oK FHM

#	176-024		500 watt lamp, 3200oK FDN

#	176-026		750 watt lamp, 3200oK EMD