## FIXTURES

### Arc Mini Ellipse

Housing shall consist of die cast and extruded aluminum components. Finish shall be high temperature black or white.

Optical Train shall consist of a type G-12 two pin lampholder accommodating 70 or 150 watt  T-6 metal halide lamp.  Unit shall have a specular double flatted reflector and plano convex lenses of low expansion glass.  Each lens system shall be continuously adjustable to achieve field angles from 30o through 50o.  An optional 20o lens shall be available.

The gate assembly shall contain four adjustable stainless steel shutters, each operating within its own independent plane.  Each shutter shall rotate plus or minus 30o within the gate for a total angular rotation between adjacent shutter blades of 120o.  These blades shall be equipped with thermally insulated handles with finger holes.

Each unit shall be provided with a pattern slot, an optional accessory holder as specified by catalog number, and a socket assembly designed for rapid filament alignment through externally fitted adjustment handles to permit peak/flat field control without use of tools.  The entire lamp assembly shall be easily removed for lamp replacement.  Each lens shall be continuously adjustable to achieve soft or hard field quality.  All operator handles shall be thermally insulated.

Socket shall be precision aligned, type G-12 and UL listed. Rated lamp seal temperatures shall not be exceeded.

A solid state, low frequency ballast shall be integrally mounted to the unit.  Ballast shall provide improved regulation to the lamp enhancing lamp performance.

Lens Housing assembly shall contain plano convex lenses mounted in die cast lens holders, secured by push-on fasteners.  The lens holders move within rails of the bottom extrusion, sliding on Torlon glides.  The hinged extruded housing allows immediate accessibility to optical components for maintenance without usage of any exterior fasteners.  The top housing (with integral handle) shall be secured by two internal spring-actuated fasteners.

Two methods of mounting shall be available. For pipe mounting the unit shall be provided with a heavy steel yoke and a painted malleable iron C-Clamp, adjustable for up to 2" ID pipe, with a tapped and threaded steel hanger pin.  A canopy mount consisting of a round face plate for 4”x4” box, threaded stud, and wire harness shall also be available.

Colorframe holder shall be located internally, with no exposed mounting surfaces.

Accessory Holder (optional) holds either 3" diameter or 4" width rectangular patterns, plus provides means for mounting the iris (customer installed).  The accessory holder shall fit directly into the pattern slot provided in each fixture.

Pipe mount unit shall be provided with 36" 3-wire VHT leads, with connector specified by catalog number.

Provide the Following:

Qty.    Catalog No. 	Description

#	8-2070	Black Arc Mini Ellipse with C-Clamp, and Parallel Blade Connector

#	8-2070W	White Arc Mini Ellipse with C-Clamp, and Parallel Blade Connector

#	OPT-00006	Fixture with Locking Connector

#	OPT-00011	Fixture with Ceiling-Mount Canopy Assembly

#	OPT-82020	Fixture with 20o  Lens Tube

#	120-011	Color Frame

#	138-010	Pattern Holder

#	138-018	Iris Kit

#	138-059	Safety Cable

#	158-003	C-Clamp for 1-2” ID Pipe

#	120-061	Donut

#	138-045	Combo Stud

#	176-310	M85, 70Watt, 3000o  K, 9000 Hour, Metal Halide Lamp