## FIXTURES

### Ellipsoidal Spotlights

Housing shall be constructed of die-cast and sheet aluminum, with a high temperature black finish.

Optical Train shall consist of a medium two-pin base socket accommodating 500, 750 or 1000 watt tungsten halogen lamp, a specular Alzak double-flatted reflector and plano-convex lens(es) of low expansion borosilicate glass, lens configuration A producing a field of B (See table below).

The gate assembly shall contain four adjustable nickel/chromium stainless steel framing shutters, each operating in its own independent plane through plus or minus 30o of rotation anywhere within the gate, with up to 120o total angular rotation between adjacent blades.  These blades shall each be equipped with thermally-insulated handles with finger holes.

Each unit shall be provided with a pattern slot and an optional customer-installed iris assembly.

The socket assembly shall be designed for rapid filament alignment by use of a thermoplastic focus handle on the socket housing permitting precise peak/flat field control. The entire assembly shall be removable without tools for lamp replacement. Units which require tools for alignment or removal of lamp assembly are not equal and are not acceptable.

Socket shall be precision type TP-22, UL recognized, steatite insulated, of die cast aluminum construction, rated for 600 volts, 1000 watts, 250oC, continuous operation.  The rated lamp seal temperatures shall not be exceeded.

Performance with a 1000 watt, 300 hour 3200oK lamp shall be C beam candlepower with a B field:

Unit

Catalog	A Lens			B				C Beam

No.		Configuration		Field				Candlepower

650-01X	2-(4.5" x 6.5")		50o				  68,500

650-02X	1-(4.5" x 9")1-(6" x 9")	40o				112,000

650-03X	1-(4.5" x 9")1-(6" x 9")	30o				154,000

650-04X	1-(6" x 9")			20o				224,000

650-05X	1-(6" x 12")			15o				308,000

650-07X	1-(8" x 14")			10o				563,000

650-08X	1-(10" x 23")		 	5o				939,000

Unit shall be provided with a heavy steel yoke, a painted malleable iron C-clamp adjustable for up to 2" ID pipe with a tapped and threaded steel hanger pin, an insulated focusing handle on the rear, and a color frame holder.  The unit shall rotate vertically through the yoke for ease of focusing.

The unit shall be provided with 36" 3-wire VHT leads, with connector as specified by catalog number.  The entire unit shall be UL listed.

Provide the Following:

Qty.    Catalog No. 	Description

#	650-0152	50o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	650-0154	50o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	650-0156	50o Ellipsoid with Locking Connector, C-Clamp, Safety Cable,

and Color Frame

#	650-0252	40o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	650-0254	40o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	650-0256	40o Ellipsoid with Locking Connector, C-Clamp, Safety Cable,

and Color Frame

#	650-0352	30o Ellipsoid with Stage Pin Connector, C-Clamp, , Safety Cable,

and Color Frame

#	650-0354	30o Ellipsoid with Parallel Blade Conn., C-Clamp, , Safety Cable,

and Color Frame

#	650-0356	30o Ellipsoid with Locking Connector., C-Clamp, , Safety Cable,

and Color Frame

#	650-0452	20o Ellipsoid with Stage Pin Connector, C-Clamp, , Safety Cable,

and Color Frame

#	650-0454	20o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	650-0456	20o Ellipsoid with Locking Connector, C-Clamp, Safety Cable,

and Color Frame

#	650-0552	15o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	650-0554	15o Ellipsoid with Parallel Blade Conn., C-Clamp Safety Cable,

and Color Frame

#	650-0556	15o Ellipsoid with Locking Connector, C-Clamp, Safety Cable,

and Color Frame

#	120-005	Color Frame - 6”

#	120-060	Donut - 6”

#	138-049	Top Hat - 6”

#	650-0752	10o Ellipsoid with Stage Pin Connector, C-Clamp, and Color Frame

#	650-0754	10o Ellipsoid with Parallel Blade Conn., C-Clamp, and Color Frame

#	650-0756	10o Ellipsoid with Locking Connector, C-Clamp, and Color Frame

#	120-015	Color Frame -8”

#	650-0852	5o Ellipsoid with Stage Pin Connector, C-Clamp, and Color Frame

#	650-0854	5o Ellipsoid with Parallel Blade Conn., C-Clamp, and Color Frame

#	650-0856	5o Ellipsoid with Locking Connector, C-Clamp, and Color Frame

#	120-016	Color Frame - 10”

General Accessories and Lamps

#	138-003	Pattern Holder

#	138-009	Iris Kit

#	138-045	Combo Stud, Flat Yoke

#	138-059	Safety Cable

#	1-9201	Glass Pattern Holder

#	176-031	500 watt lamp, 3000oK EHD

#	176-200	FLK 575Watt, 115V, 3200oK, 300HR, HX-600

#	176-601	575Watt, 115V, 3050oK, 2000HR, HX-601

#	176-033	1000 watt lamp, 3200oK FEL

#	176-185	750 watt lamp, 3200oK EHF

#	176-187	750 watt lamp, 3000oK EHG