## FIXTURES

### 6” Television Fresnels

Housing shall consist of die cast and extruded aluminum components with high temperature black finish.

The Optical Train shall consist of medium bipost tungsten halogen lamp, an etched super pure aluminum clad reflector, 6" x 4" low expansion borosilicate fresnel lens mounted in a hinged door for rapid lamp replacement.

The Socket shall be medium bipost with steatite insulators and beryllium copper contacts.It shall be UL recognized for 240 volts, 1000 watts, continuous operation.  Rated lamp seal temperatures shall not be exceeded.

Performance with 176-097 (ANSI code EGT) lamp in spot focus shall be 184,000 beam candle power with 13.9o field, and in flood focus shall be 16,200 beam candle power with 68.8o field.

For mounting, unit shall be provided with a heavy steel yoke and a painted malleable iron C-clamp adjustable for up to 2" I.D. pipe, with a tapped and threaded steel hanger pin. Accessories shall be mounted into front door gel clips and secured with a positive locking safety clip.

Unit shall be provided with 36" long 3 wire leads covered by black sleeving and connector as specified by catalog number.

Provide the Following:

Qty.    Catalog No. 	Description

#	100-512	6" Theatre Fresnel with Stage Pin Connector

#	100-514	6" Theatre Fresnel with Parallel Blade Connector

#	100-516	6" Theatre Fresnel with Locking Connector

#	118-013 	Eight Leaf Barndoor

#	120-005 	Color Frame

#	122-147 	Set, Single,Half Single Scrims

#	130-003 	Dichroic Filter

#	138-045 	Combo Stud for flat yokes

#	138-049 	6" High Hat

#	138-059 	Safety Cable

#	176-167 	500 watt lamp, 3050oK BTL

#	176-168 	500 watt lamp, 3200oK BTM

#	176-102 	500 watt lamp, 3200oK EGN

#	176-147 	750 watt lamp, 3200oK EGR

#	176-097 	1000 watt lamp, 3200oK EGT