## FIXTURES

### 8” Television Fresnels

Housing shall consist of die cast and extruded aluminum components with high temperature black finish.

The Optical Train shall consist of mogul bipost tungsten halogen lamp, an etched super pure aluminum clad reflector, 8" x 5" low expansion borosilicate fresnel lens mounted in a hinged door for rapid lamp replacement.

The Socket  shall be mogul bipost with diecast aluminum housing, steatite insulators, copper contacts. It shall be UL recognized for 240 volts, 10,000 watts, continuous operation.  Rated lamp seal temperatures shall not be exceeded.

Performance with 176-075 (ANSI code CYX) lamp in spot focus shall be 366,000 beam candle power with 15.1o field, and in flood focus shall be 39,500 beam candle power with 67.0o field.

For mounting, unit shall be provided with a heavy steel yoke and a painted malleable iron C-clamp adjustable for up to 2" I.D. pipe, with a tapped and threaded steel hanger pin. Accessories shall be mounted into front door gel clips and secured with a positive locking safety clip.

Unit shall be provided with 36" long 3 wire leads covered by black sleeving and connector as specified by catalog number.

Provide the Following:

Qty.    Catalog No. 	Description

#	100-522	 8" Television Fresnel with Stage Pin Connector

#	100-524	 8" Television Fresnel with Parallel Blade Connector

#	100-526	 8" Television Fresnel with Locking Connector

#	118-032 	Eight Leaf Barndoor

#	120-015 	Color Frame

#	122-148 	Set, Single, Half Single Scrims

#	138-045	 Combo Stud for flat yokes

#	138-051 	8" High Hat

#	138-059 	Safety Cable

#	176-161	1000 watt lamp, 3200oK CYV

# 	176-157 	1500 watt lamp, 3200oK CXZ

# 	176-075 	2000 watt lamp, 3200oK CYX