## FIXTURES

### 6”  Theatre Fresnels

Housing shall consist of die cast and extruded aluminum components with high temperature black finish.

The Optical Train shall consist of medium prefocus tungsten halogen lamp, an etched super pure aluminum clad reflector, 6" x 4" low expansion borosilicate fresnel lens mounted in a hinged door for rapid lamp replacement.

The Socket shall be medium prefocus, porcelain insulated, UL recognized for 250 volts, 1200 watts, 200oC continuous operation.  Rated lamp seal temperatures shall not be exceeded.

Performance with 176-171 (ANSI code BTR) lamp in spot focus shall be 175,000 beam candle power with 12.5o field, and in flood focus shall be 11,200 beam candle power with 63.5o field.

For mounting, unit shall be provided with a heavy steel yoke and a painted malleable iron C-clamp adjustable for up to 2" I.D. pipe, with a tapped and threaded steel hanger pin. Accessories shall be mounted into front door gel clips and secured with a positive locking safety clip.

Unit shall be provided with 36" long 3 wire leads covered by black sleeving and connector as specified by catalog number.

Provide the Following:

Qty.    Catalog No. 	Description

#	213-512	6" Theatre Fresnel with Stage Pin Connector

#	213-514	6" Theatre Fresnel with Parallel Blade Connector

#	213-516	6" Theatre Fresnel with Locking Connector

#	118-013 	Eight Leaf Barndoor

#	120-005 	Color Frame

#	122-147 	Set, Single,Half Single Scrims

#	130-003 	Dichroic Filter

#	138-045 	Combo Stud for flat yokes

#	138-049 	6" High Hat

#	138-059 	Safety Cable

#	176-167 	500 watt lamp, 3050oK BTL

#	176-168 	500 watt lamp, 3200oK BTM

#	176-169 	750 watt lamp, 3050oK BTN

#	176-170 	750 watt lamp, 3200oK BTP

#	176-171 	1000 watt lamp, 3200oK BTR