## FIXTURES

### 8” Theatre Fresnels

Housing shall consist of die cast and extruded aluminum components with high temperature black finish.

The Optical Train shall consist of mogul prefocus tungsten halogen lamp, an etched super pure aluminum clad reflector, 8" x 5" low expansion borsilicate fresnel lens mounted in a hinged door for rapid lamp replacement.

The Socket  shall be mogul prefocus, porcelain insulated, UL recognized for 250 volts, 2500 watts, 200oC continuous operation.  Rated lamp seal temperatures shall not be exceeded.

The Performance with 176-194 (ANSI code BVW) lamp in spot focus shall be 358,000 beam candle power with 15.1o field, and in flood focus shall be 37,000 beam candle power with 63.5o field.

For mounting, unit shall be provided with a heavy steel yoke and a painted malleable iron C-clamp adjustable for up to 2" I.D. pipe, with a tapped and threaded steel hanger pin. Accessories shall be mounted into front door gel clips and secured with a positive locking safety clip.

Unit shall be provided with 36" long 3 wire leads covered by black sleeving and connector as specified by catalog number.

Provide the Following:

Qty.    Catalog No. 	Description

#	213-522	 8" Theatre Fresnel with Stage Pin Connector

#	213-524	 8" Theatre Fresnel with Parallel Blade Connector

#	213-526	 8" Theatre Fresnel with Locking Connector

#	118-032 	Eight Leaf Barndoor

#	120-015 	Color Frame

#	122-148 	Set, Single, Half Single Scrims

#	138-045	 Combo Stud for flat yokes

#	138-051 	8" High Hat

#	138-059 	Safety Cable

#	176-191 	1000 watt lamp, 3050oK BVT

#	176-192	1000 watt lamp, 3200oK BVV

#	176-193 	1500 watt lamp, 3200oK CWZ

#	176-194 	2000 watt lamp, 3200oK BVW