## FIXTURES

### Ellipsoidal Spotlights

The instrument shall be a LEO ellipsoidal spotlight as manufactured by Leviton, Mfg. or approved equal.

The housing shall be constructed of rugged, die-cast aluminum, finished in durable, textured black powder coat.

Optical Train shall consist of a medium two-pin base socket accommodating 500, 750 or 1000 watt tungsten halogen lamp, the reflector shall be faceted glass with a dichroic coating, and plano-convex lenes of low expansion borosilicate glass.

The lenses and reflector shall be insulated from the housing. The fixture shall employ Quick-Center, locking lamp centering technology allowing the lamp centering focus to remain constant when changing lamps of identical type.  Peak/Cosine field adjustment shall be performed separately from lamp centering.  Removal of lamp housing shall be tool free and be performed without disturbing the focus or alignment for ease of lamp replacement.  It shall be possible to possible to replace a lamp in 30-seconds with no focus readjustments required.

Removable, interchangeable lens tubes shall provide a variety of focal degrees as follows; 15, 19, 26, 36,  or 50 degree field angles.  It shall be possible to obtain

Cosine (flat) and peak/blending focus with all recommended lamps and beam angles.

The fixture shall employ a system whereby lens can be field repositioned to achieve different beam angles with the same lens tube.  Units not providing this capability shall not be considered acceptable.

Efficacy of reflector shall be 96%. Reflector shall absorb 91% of the heat produced from the lamp.  Fixtures which do not meet this requirement shall not be acceptable.

The fixture shall have body and barrel on-axis rotation of 360°.  The shutters shall be made of heavy-gage stainless steel and arranged on four independent planes. Fixtures offering limited range of motion are not equal.  Shutters shall be field replaceable without requiring tools or fixture disassembly.

Two accessory slots at shall be available at a 3” diameter gate. The fixture shall support glass and steel gobos, size A or B, in the 3” gate.  It shall be possible to use motorized gobo rotators, iris, gobo holders, and other standard Ellipsoidal accessories.

The fixture shall employ Easy Glide lens tubes, providing no metal to metal contact for ease of movement.  Ease of movement shall not be affected by the temperature of the unit.  Two integral, die-cast locking gel/media/accessory slots shall be available.

The fixture shall employ a sturdy steel yoke allowing multiple mounting positions and be provided with a painted malleable iron C-clamp adjustable for up to 2” ID pipe.  The unit shall rotate vertically through the yoke for ease of focusing.

A cool grip focusing handle shall be provided on the rear of the unit which also serves as a wire wrap when spotlight is stored. The fixture shall employ insulated Cool Grip handles on all operating surfaces.  All parts shall be user replaceable.

The unit shall accommodate lamps up to 750 watts utilizing HX series and other specifically listed lamps. The fixture shall be rated at 115-240 volts and be UL and cUL listed.  36 inch long sleeved 3-wire VHT electrical leads shall be provided with connector as specified by catalog number.

The manufacturer of the fixtures must publish Photometric Performance data in conformance with ANSI standard E1.9-2001 or a more recent standard.  Manufacturers which do not publish data per this standard shall not be acceptable.

Provide the Following:

Qty.    Catalog No. 	Description

#	lel50-osb	50o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel50-opb	50o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	lel50-otb	50o Ellipsoid with Twistlock Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel50-oob	50o Ellipsoid with Bear Leads, C-Clamp, Safety Cable,

and Color Frame

#	lel36-osb	36o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel36-opb	36o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	lel36-otb	36o Ellipsoid with Twistlock Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel36-oob	26o Ellipsoid with Bear Leads, C-Clamp, Safety Cable,

and Color Frame

#	lel26-osb	26o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel26-opb	26o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	lel26-otb	26o Ellipsoid with Twistlock Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel26-oob	26o Ellipsoid with Bear Leads, C-Clamp, Safety Cable,

and Color Frame

#	lel19-osb	19o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel19-opb	19o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	lel19-otb	19o Ellipsoid with Twistlock Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel19-oob	19o Ellipsoid with Bear Leads, C-Clamp, Safety Cable,

and Color Frame

#	lel15-osb	15o Ellipsoid with Stage Pin Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel15-opb	15o Ellipsoid with Parallel Blade Conn., C-Clamp, Safety Cable,

and Color Frame

#	lel15-otb	15o Ellipsoid with Twistlock Connector, C-Clamp, Safety Cable,

and Color Frame

#	lel15-oob	15o Ellipsoid with Bear Leads, C-Clamp, Safety Cable,

and Color Frame

General Accessories and Lamps

#	TBD		Color Frame - 6”

#	TDB		Donut - 6”

#	TBD		Top Hat - 6”

#	TBD		Pattern Holder

#	TBD		Iris Kit

#	138-045	Combo Stud, Flat Yoke

#	138-059	Safety Cable

#	TBD		Glass Pattern Holder

#	LOGLC-000	575 Watt, 115V, 3200oK, 300HR, GLC

#	LOGLA-000	575 Watt, 115V, 3050oK, 1500HR, GLA

#	LOGLD-000	750 Watt, 115V, 3200oK, 300HR, GLD

#	LOGLE-000	750 Watt, 115V, 3050oK, 1500HR, GLE

#	LOGKV-000	600 Watt, 240V, 3200oK, 250HR, GKV

#	LOGKV-LL0  600 Watt, 240V, 3000oK, 1500HR, GKV/LL