## FIXTURES

### Mini Soft-Lite

Housing shall be constructed of aluminum and steel with a baked enamel grey and black enamel finish.

Optical Train shall consist of recessed single contact double ended base tungsten halogen lamps, high temperature diffused Alzak reflector and easily accessible socket assemble for lamp replacement.

Socket assembly shall be UL listed, steatite insulated and rated for 600 volts, 3000 watts, 200oC continuous operation. Rated lamp seal temperatures shall not be exceeded.

Performance with 2 catalog number 176-021 (ANSI code FCM) lamps in a 66 degree beam and a 110 degree field is 11, 200 beam candlepower.

For mounting, unit shall be provided with a heavy steel yoke with combo stud - fits 5/8" stud or 1 1/8" female.

Unit shall be provided with cable and connector as specified by catalog number.  The entire unit shall be UL listed and  labeled.

Two 10 amp T-rated switches in housing are provided for individual control of lamps.

Voltage:  120/240 AC/DC.

Amperage:	16.6 amps at 120V with two 1000 watt lamps.

Provide the Following:

Qty.    Catalog No. 	Description

#	104-171	Mini Soft-Lite with 3 wire parallel blade, U-ground cap

#	120-021	Diffusion frame

#	138-059	Safety cable

#	152-053	Compact Stand, Castered

#	176-019	500 watt lamp, 3200oK EJD

#	176-021	1000 watt lamp, 3200oK FCM